Supplementary File

From Food Waste to Value: Robust Superstructure Optimization for a Circular Economy

Harriet D. Appiah1†, Russel E. Vistal2‡, Jagadish Torlapati3‡, and Kirti M. Yenkie4†*

## † Rowan University, Department of Chemical Engineering, Glassboro, NJ 08028, USA

## ‡ Rowan University, Department of Civil and Environmental Engineering, Glassboro, NJ 08028, USA

## * Corresponding Author: yenkie@rowan.edu

S1. Model Structure

The superstructure shown in Figure S1 (Figure 1 in the manuscript) consists of three stages: pretreatment, conversion, and product recovery. Stage I includes mechanical pretreatment using either a shredder (SHR) or wet macerator (MCR), followed by biological pretreatment using aerobic pretreatment (AER) or enzymatic hydrolysis (ENZ). Bypass options allow the feed to skip the mechanical or biological pretreatment steps. In Stage II, the pretreated feed can be distributed among hydrothermal liquefaction (HTL), anaerobic digestion (AND), sanitary landfill (SLF), composting (CMP), wastewater treatment (WWT), and incineration (INC). Stage III includes centrifugation (CEN) and filtration (FLT) for biocrude recovery from HTL, and amine absorption (ABS) and pressure swing adsorption (PSA) for biomethane recovery from the gas-producing pathways. Incineration is coupled with a steam turbine (STB) for electricity generation. The final products represented in the superstructure are biocrude, biomethane, compost, biosolids, and electricity.

Figure 1: Food waste valorization superstructure showing the stream numbering used in the model equations: Stage I, pretreatment; Stage II, conversion; and Stage III, product recovery and upgrading

Pretreatment technologies are selected using binary variables because a single facility builds at most one mechanical and one biological pretreatment line for the entire feed. Bypass streams allow either or both pretreatment steps to be omitted. The conversion, product recovery, and biogas upgrading pathways are represented by continuous split fractions bounded between 0 and 1, allowing the feed to be distributed among multiple pathways simultaneously. Each unit is sized according to the material flow routed to it. The model was implemented in GAMSPy (version 1.21.0) and solved with BARON 25.12.10 through GAMS 53.1.0. Technologies, streams, and components are indexed by i, j, and k, respectively. The notebook includes feed characterization, process and yield parameters, variable bounds, unit operation equations, and economic and environmental calculations. The final blocks solve the model and generate the payoff table, ε-constraint solutions, and figures.

S2. Mathematical Modeling

The mathematical formulation is presented in this section. The sets and indices used throughout the formulation are defined first, followed by the equations describing each unit operation. The model consists of component mass balances, technology-specific performance equations, capacity and utility requirements, economic equations, and environmental impact equations.

S2.1. Sets and Indices

The model considers technologies i, material streams j, stream components k, and candidate solutions p generated during the ε-constraint analysis. The corresponding sets are defined as follows:

i∈I - Technologies (used as subscript to variables)

{SHR: shredder,

MCR: wet macerator,

AER: aerobic pretreatment,

ENZ: enzymatic hydrolysis,

HTL: hydrothermal liquefaction,

AND: anaerobic digestion,

SLF: sanitary landfill,

CMP: composting,

WWT: wastewater treatment,

INC: incineration,

CEN: centrifuge,

FLT: filtration,

ABS: absorption,

PSA: pressure swing adsorption,

STB: steam turbine}

Splitters 1 to 5 and Mixers 1 to 5 connect the technologies; BYP1, BYP2 and BYP3 are bypass routes.

j∈J - streams (used as subscripts to variables)

{1, 2, 3, …, 61}

k∈K - stream components (used as subscripts to variables)

{CBH: carbohydrate, PRT: protein, FAT: lipid, OTH: other volatile organics, WATER: water, FC: fixed carbon, ASH: ash, BIOCRUDE: biocrude, CHAR: hydrochar, AQ: HTL aqueous phase, GAS: HTL gas product, COMPOST: compost, BIOSOLIDS: biosolids, ELECTRICITY: electricity, CO2, CH4, N2O, SO2, N2, O2, NH3, H2S, ENZYME: enzyme}

p∈P - candidate solutions of the ε-constraint grid

{P1, P2, …, P25}

S2.1.1. Subsets

Subsets for technologies

IMECH - mechanical pretreatment

{SHR, MCR}

IBIO - biological pretreatment

{AER, ENZ}

ICONV - conversion technologies

{HTL, AND, SLF, CMP, WWT, INC}

IREC - HTL product recovery

{CEN, FLT}

IUPG - biogas upgrading

{ABS, PSA}

Subsets for components

KVS - volatile solids

{CBH, PRT, FAT, OTH}

KHTL - HTL products

{BIOCRUDE, CHAR, AQ}

KP - saleable products

{CH4, BIOCRUDE, COMPOST, ELECTRICITY, BIOSOLIDS}

S2.2. Technology design model equations

The equations describing the material flows, process performance, capacity requirements, and utility demands of each technology are presented below.

S2.2.1 Feed, splitters and mixers

The feed composition is specified by the total food waste feed rate, Qf, and the mass fraction, Xk, of each component k. The stream numbers follow Figure S1.

Feed assignment:

|  | M1,k=QfXk ;∀k∈K | (1) |
| --- | --- | --- |

Splitter 1 distributes the food waste among the mechanical-pretreatment bypass (BYP1, stream 2), shredding (stream 3),  maceration (stream 6) and the direct-conversion bypass (BYP2, stream 58).

Splitter 1 balance:

|  | M1,k=M2,k+M3,k+M6,k+M58,k ;∀k∈K | (2) |
| --- | --- | --- |

Splitter 1 flow activation:

|  | M2,k≤QfXkyBYP1, M3,k≤QfXkySHR ;∀k∈K | (3) |
| --- | --- | --- |
|  | M6,k≤QfXkyMCR, M58,k≤QfXkyBYP2 ;∀k∈K | (4) |

Mechanical pretreatment selection:

|  | ySHR+yMCR+yBYP1+yBYP2=1 | (5) |
| --- | --- | --- |

Mixer 1:

|  | M9,k=M4,k+M8,k ;∀k∈K | (6) |
| --- | --- | --- |

Splitter 2 receives the mechanically pretreated stream, M9,k, and the BYP1 stream, M2,k, and distributes the material among aerobic pretreatment (stream 10), enzymatic hydrolysis (stream 14) and the biological-pretreatment bypass (BYP3, stream 61).

Splitter 2 balance:

|  | M9,k+M2,k=M10,k+M14,k+M61,k ;∀k∈K | (7) |
| --- | --- | --- |

Splitter 2 flow activation:

|  | M10,k≤MbigyAER, M14,k≤MbigyENZ, M61,k≤MbigyBYP3 ;∀k∈K | (8) |
| --- | --- | --- |

Biological pretreatment selection:

|  | yAER+yENZ+yBYP3=1-yBYP2 | (9) |
| --- | --- | --- |

Mixer 2:

|  | M18,k=M12,k+M17,k+M61,k ;∀k∈K | (10) |
| --- | --- | --- |

Splitter 3 combines the material leaving the pretreatment stage with the direct-conversion bypass and distributes it among the six conversion technologies.

Splitter 3 balance:

|  | M18,k+M58,k=M19,k+M23,k+M28,k+M31,k+M35,k+M38,k ;∀k∈K | (11) |
| --- | --- | --- |

Splitter 3 split to the conversion technologies:

|  | Mji,k=fiCONVM18,k+M58,k ;∀i∈ICONV,k∈K | (12) |
| --- | --- | --- |

where

ji=19,  i=HTL23,  i=AND28,  i=SLF31,  i=CMP35,  i=WWT38,  i=INC                                                                                 (13)

|  | i∈ICONV​fiCONV=1, 0≤fiCONV≤1 ;∀i∈ICONV | (14) |
| --- | --- | --- |

fiCONV is the fraction of the material entering Splitter 3 that is routed to conversion technology i.

Only landfill and incineration may receive food waste that has not been mechanically pretreated:

|  | yBYP1+yBYP2≤1-fWWTCONV-fANDCONV-fHTLCONV-fCMPCONV | (15) |
| --- | --- | --- |

Splitter 4 divides the HTL product slurry (stream 22) between centrifugation (stream 41) and filtration (stream 44). fiREC is the fraction of stream 22 sent to recovery technology i.

Splitter 4 balance and split:

|  | M22,k=M41,k+M44,k ;∀k∈K | (16) |
| --- | --- | --- |
|  | M41,k=fCENRECM22,k, M44,k=fFLTRECM22,k ;∀k∈K | (17) |
|  | i∈IREC​fiREC=1, 0≤fiREC≤1 ;∀i∈IREC | (18) |

Mixer 3 (AND biogas and captured landfill gas):

|  | M48,k=M27,k+M30,k ;∀k∈K | (19) |
| --- | --- | --- |

Splitter 5 divides the combined gas (stream 48) between absorption (stream 49) and pressure swing adsorption (stream 53). fiUPG is the fraction of stream 48 sent to upgrading technology i.

Splitter 5 balance and split:

|  | M48,k=M49,k+M53,k ;∀k∈K | (20) |
| --- | --- | --- |
|  | M49,k=fABSUPGM48,k, M53,k=fPSAUPGM48,k ;∀k∈K | (21) |
|  | i∈IUPG​fiUPG=1, 0≤fiUPG≤1 ;∀i∈IUPG | (22) |

Mixer 4 (biocrude from CEN and FLT) and biocrude product:

|  | M47,k=M43,k+M46,k ;∀k∈K | (23) |
| --- | --- | --- |
|  | MHTL,BIOCRUDEP=M47,BIOCRUDE | (24) |

Mixer 5 (biomethane from ABS and PSA) and biomethane product:

|  | M56,k=M52,k+M55,k ;∀k∈K | (25) |
| --- | --- | --- |
|  | MABS,CH4P+MPSA,CH4P=M56,CH4 | (26) |

Steam turbine operates on the incinerator flue gas:

|  | fSTB=fINCCONV | (27) |
| --- | --- | --- |

Design capacity of every technology:

|  | QiC≥QiS ;∀i∈I | (28) |
| --- | --- | --- |

Mbig is the upper bound on any stream flow (big-M).

Shredder (SHR)

Unit Specific Parameters

ηSHR: mass fraction of the feed passed to product

WSHRsp: specific power of the shredder (kW per kg/h)

Variables

QSHRS: Capacity (kg/h)

PWSHR: operating power (kW)

Equations

Product stream:

|  | M4,k=ηSHRM3,k ;∀k∈K | (29) |
| --- | --- | --- |

Reject stream:

|  | M5,k=1-ηSHRM3,k ;∀k∈K | (30) |
| --- | --- | --- |

Capacity:

|  | QSHRS=k∈K​M3,k | (31) |
| --- | --- | --- |

Power required:

|  | PWSHR=WSHRspQSHRS | (32) |
| --- | --- | --- |

Macerator (MCR)

Unit Specific Parameters

TSMCR: target total solids fraction of the pumpable slurry

WMCRsp: specific power of the macerator (kW per kg/h)

Variables

M7,WATER: dilution water added (kg/h)

QMCRS: capacity (kg/h)

PWMCR: operating power (kW)

Equations

Dry solids entering the macerator:

|  | MDS,MCR=k∈K,k≠WATER​M6,k | (33) |
| --- | --- | --- |

Dilution water to reach the target solids content:

|  | M7,WATER=1TSMCR-1MDS,MCR-M6,WATER | (34) |
| --- | --- | --- |

Slurry stream:

|  | M8,k=M6,k+M7,k ;∀k∈K | (35) |
| --- | --- | --- |

Capacity:

|  | QMCRS=k∈K​M6,k | (36) |
| --- | --- | --- |

Power required:

|  | PWMCR=WMCRspQMCRS | (37) |
| --- | --- | --- |

Aerobic Pretreatment (AER)

fdegAER: fraction of volatile solids degraded

EARAER: excess air ratio

yO2, yN2: O₂ and N₂ mass fractions of air

YO2AER, YCO2AER: O₂ demand and CO₂ yield per kg VS degraded, calculated from the elemental composition of the feed (kg/kg VS)

ρs, ρw: solids and water densities (kg/m³)

θAER: residence time  (h)

ϵAER: working volume fraction

WAERsp: specific mixing power (kW/m³)

Variables

VSdegAER: volatile solids degraded  (kg/h)

O2AER: oxygen consumed (kg/h)

VwAER, VAER: working and vessel volumes (m³)

QAERS: vessel volume used for costing (m³)

PWAER: operating power (kW)

Equations

Volatile solids degraded:

|  | VSdegAER=k∈KVS​fdegAERM10,k | (38) |
| --- | --- | --- |

Oxygen consumed:

|  | O2AER=YO2AER VSdegAER | (39) |
| --- | --- | --- |

Aeration air:

|  | M13,O2=EARAERO2AER | (40a) |
| --- | --- | --- |
|  | M13,N2=yN2yO2M13,O2 | (41b) |

Undegraded organics in  product stream:

|  | M12,k=1-fdegAERM10,k ;∀k∈KVS | (42) |
| --- | --- | --- |

Off-gas CO₂, O₂ and N₂:

|  | M11,CO2=YCO2AERVSdegAER | (43a) |
| --- | --- | --- |
|  | M11,O2=M13,O2-O2AER | (43b) |
|  | M11,N2=M13,N2 | (43c) |

Working volume:

|  | VwAER=θAERk≠WATER​M10,kρs+M10,WATERρw | (44) |
| --- | --- | --- |

Vessel volume:

|  | ϵAERVAER=VwAER | (45) |
| --- | --- | --- |

Capacity:

|  | QAERS=VAER | (46) |
| --- | --- | --- |

Power required:

|  | PWAER=WAERspQAERS | (47) |
| --- | --- | --- |

Enzymatic Hydrolysis (ENZ)

Unit Specific Parameters

rENZ: enzyme dose per kg dry solids (kg/kg)

cp: heat capacity of the feed (kJ/(kg K))

TENZ, Tamb: hydrolysis and ambient temperatures (°C)

Δhstm: latent heat of steam (kJ/kg)

ρs, ρw: solids and water densities (kg/m³)

θENZ: hydraulic residence time (h)

ϵENZ: working volume fraction

WENZsp: specific power (kW/m³)

ηAND: base AND volatile solids destruction

δENZ: uplift of AND destruction when ENZ is upstream

Variables

DSENZ: dry solids entering (kg/h)

VENZ: vessel volume (m³)

VwENZ: working volume (m³)

HENZ: heat duty (kJ/h)

MENZstm: steam demand (kg/h)

QENZS: vessel volume used for costing (m³)

PWENZ: operating power  (kW)

ηeffAND: effective AND destruction efficiency

Equations

Dry solids entering:

|  | DSENZ=k∈K,k≠WATER​M14,k | (348 |
| --- | --- | --- |

Enzyme addition:

|  | M15,ENZYME=rENZDSENZ | (49) |
| --- | --- | --- |

Product stream:

|  | M17,k=M14,k+1k=PRT M15,ENZYME ;∀k∈K,k≠ENZYME | (50a) |
| --- | --- | --- |

where 1k=PRT equals 1 for protein and 0 otherwise.

Off-gas stream:

|  | M16,k=0 ;∀k∈K | (51) |
| --- | --- | --- |

Heat to raise the feed to the hydrolysis temperature:

|  | HENZ=k∈K​M14,k+M15,ENZYMEcpTENZ-Tamb | (52) |
| --- | --- | --- |

Steam demand:

|  | MENZstm=HENZΔhstm | (53) |
| --- | --- | --- |

Working volume:

|  | VwENZ=θENZk∈K,k≠WATER​M14,k+M15,ENZYMEρs+M14,WATERρw | (54) |
| --- | --- | --- |

Vessel volume:

|  | ϵENZVENZ=VwENZ | (55) |
| --- | --- | --- |

Capacity:

|  | QENZS=VENZ | (56) |
| --- | --- | --- |

Power required:

|  | PWENZ=WENZspQENZS | (57) |
| --- | --- | --- |

Annual enzyme cost:

|  | CENZenz=cenzTannM15,ENZYME×10-6 | (58) |
| --- | --- | --- |

Effect on anaerobic digestion:

|  | ηeffAND=ηAND1+δENZyENZ | (59) |
| --- | --- | --- |

Hydrothermal Liquefaction (HTL

Unit Specific Parameters

TSHTL: design solids content of the HTL feed

YkHTL: yield of products(kg/kg dry ash-free organics)

ρHTL: feed slurry density at inlet conditions (kg/m³)

θHTL: residence time  (h)

ϵHTL: working volume fraction

ηHX: heat recovery

Δhw: enthalpy rise of the feed from ambient to reaction temperature at reactor pressure (kJ/kg)

HHVgas: heating value of the HTL gas (MJ/kg)

LHVNG: lower heating value of natural gas (MJ/kg)

ηfh: fired heater efficiency

wN: nitrogen mass fraction of the feed (dry, ash free)

ξAQN: fraction of HTL feed nitrogen reporting to the aqueous phase

WHTLsp: specific power (kW/m³)

Variables

MHTLdaf: dry ash-free organics entering (kg/h)

mHTL: heated slurry mass (kg/h)

VwHTL, VHTL: working and vessel volumes (m³)

HnetHTL, HgasHTL: net heat duty and heat from burning the HTL gas (kJ/h)

MHTLNG: natural gas burned in the fired heater (kg/h)

NAQ: nitrogen to the aqueous phase  (kg N/h)

PWHTL: operating power (kW)

Equations

Dry ash-free organics:

|  | MHTLdaf=k∈KVS​M19,k+M19,FC | (60) |
| --- | --- | --- |

Dilution water to the design solids content:

Water already present in the HTL feed + added water must be at least the water needed to reach the design solids content of the dry material entering HTL

|  | M19,WATER+M21,WATER≥1TSHTL-1k≠WATER​M19,k | (61) |
| --- | --- | --- |

HTL gas Stream:

|  | M20,GAS=YGASHTLMHTLdaf | (62) |
| --- | --- | --- |

Biocrude stream:

|  | M22,BIOCRUDE=YBIOCRUDEHTLMHTLdaf | (63) |
| --- | --- | --- |

Char Stream:

|  | M22,CHAR=YCHARHTLMHTLdaf+M19,ASH | (64) |
| --- | --- | --- |

Aqueous phase:

|  | M22,AQ=YAQHTLMHTLdaf+M19,WATER+M21,WATER | (65) |
| --- | --- | --- |

Nitrogen to the aqueous phase:

|  | NAQ=wNMHTLdaf ξAQN | (66) |
| --- | --- | --- |

Heated mass:

|  | mHTL=k​M19,k+M21,WATER | (67) |
| --- | --- | --- |

Working volume:

|  | ρHTLVwHTL=θHTLmHTL | (68) |
| --- | --- | --- |

Vessel volume:

|  | ϵHTLVHTL=VwHTL | (69) |
| --- | --- | --- |

Capacity:

|  | QHTLS=VHTL | (70) |
| --- | --- | --- |

Power required:

|  | PWHTL=WHTLspQHTLS | (71) |
| --- | --- | --- |

Net heat duty after heat recovery:

|  | HnetHTL=1-ηHXk∈K​M19,k+M21,WATERΔhw | (72) |
| --- | --- | --- |

Heat from burning the HTL gas:

|  | HgasHTL=1000 HHVgasM20,GAS | (73) |
| --- | --- | --- |

Natural gas for the fired heater:

|  | 1000 LHVNGηfhMHTLNG≥HnetHTL-HgasHTL | (74) |
| --- | --- | --- |

Anaerobic Digestion (AD)

Unit Specific Parameters

TSAND: design solids content of the digester feed

ηAND: volatile solids destruction

ηOTHAND: fraction of other volatile organics (OTH) available for digestion

YgAND: Buswell yield of g ∈ {CH4, CO2, NH3, H2S} and water consumed per kg VS destroyed, calculated from the elemental composition of the feed (kg/kg VS)

ηcapAND: fraction of generated biogas captured

θAND: hydraulic residence time (h)

ϵAND: working volume fraction

ρs, ρw: solids and water densities (kg/m³)

cp: heat capacity of the feed (kJ/(kg K))

TAND, Tamb: digester and ambient temperatures (°C)

Δhstm: latent heat of steam (kJ/kg)

WANDsp: specific mixing power (kW/m³)

Variables

VSinAND, VSdegAND: digestible volatile solids entering and destroyed (kg/h)

GgAND: mass of g generated (kg/h)

ηeffAND: effective volatile solids destruction (Enzymatic Hydrolysis section)

VwAND, VAND: working and vessel volumes (m³)

QANDS: vessel volume used for costing (m³)

MANDstm: steam demand (kg/h)

PWAND: operating power (kW)

Equations

Water  added to achieve the design solids content:

|  | M23,WATER+M24,WATER=1TSAD-1k≠WATER​M23,k | (75) |
| --- | --- | --- |

Volatile solids entering:

|  | VSinAD=M23,CBH+M23,PRT+M23,FAT+ηOTHADM23,OTH | (76) |
| --- | --- | --- |

Volatile solids destroyed:

|  | VSdegAD=ηeffADVSinAD | (77) |
| --- | --- | --- |

Buswell based gas generation:

|  | GgAD=YgADVSdegAD ;g∈{CH4,CO2,NH3,H2S,H2O} | (78) |
| --- | --- | --- |

Captured biogas:

|  | M27,g=ηcapADGgAD ;g∈{CH4,CO2,NH3,H2S,H2O} | (79) |
| --- | --- | --- |

Uncaptured gas:

|  | M25,g=1-ηcapADGgAD ;g∈{CH4,CO2,NH3,H2S,H2O} | (80) |
| --- | --- | --- |

Digestate, undegraded organics:

|  | M26,k=1-ηeffADM23,k ;k∈KVS | (81) |
| --- | --- | --- |

Digestate, ash, and fixed carbon:

|  | M26,k=M23,k ;k∈{ASH,FC} | (82) |
| --- | --- | --- |

Working volume:

|  | VwAND=θANDk≠WATER​M23,kρs+M23,WATER+M24,WATERρw | (83) |
| --- | --- | --- |

Vessel volume:

|  | ϵANDVAND=VwAND | (84) |
| --- | --- | --- |

Capacity:

|  | QANDS=VAND | (85) |
| --- | --- | --- |

Steam to heat the feed to digester temperature:

|  | MANDstm=k∈K​M23,k+M24,WATERcpTAND-TambΔhstm | (86) |
| --- | --- | --- |

Power required:

|  | PWAD=WADspQANDS | (87) |
| --- | --- | --- |

Sanitary Landfill (SLF)

Unit Specific Parameters

DOC, DOCf: degradable organic carbon and fraction dissimilated (IPCC 2006)

MCF: methane correction factor

F: CH₄ volume fraction of landfill gas

γCH4/C: CH₄ to carbon mass ratio (kg/kg)

MWCO2, MWCH4: molecular weights (kg/kmol)

CAP, OX: gas capture and cover oxidation

Tann: operating hours (h/yr)

ρFW: food waste solids density (kg/m³)

DSLF: landfill depth  (m)

Aacre: area conversion  (m²/acre)

WSLFsp : specific power (kW/acre)

Variables

GCH4SLF, GCO2SLF: landfill gas generation (kg/h)

GoxSLF: methane oxidized in the cover (kg/h)

mleftSLF: permanently buried residue  (kg/h)

VSLF: annual landfill volume (m³/yr)

QSLFS: new land required (acre/yr)

PWSLF: operating power (kW)

Equations

Methane generated:

|  | GCH4SLF=DOC DOCf MCF F γCH4/Ck∉{WATER,ASH}​M28,k | (67) |
| --- | --- | --- |

Carbon dioxide generated:

|  | GCO2SLF=GCH4SLF(1-FF)MWCO2MWCH4 | (68) |
| --- | --- | --- |

Methane oxidized in the cover:

|  | GoxSLF=1-CAPOXGCH4SLF | (69) |
| --- | --- | --- |

Captured gas stream:

|  | M30,g=CAPGgSLF ;g∈{CH4,CO2} | (70) |
| --- | --- | --- |

Uncaptured methane stream :

|  | M29,CH4=1-CAP1-OXGCH4SLF | (71) |
| --- | --- | --- |

Uncaptured carbon dioxide stream:

|  | M29,CO2=1-CAPGCO2SLF+MWCO2MWCH4GoxSLF | (72) |
| --- | --- | --- |

Residue(Wasteleft):

|  | mleftSLF=k​M28,k-g∈{CH4,CO2}​M29,g+M30,g | (73) |
| --- | --- | --- |

Annual landfill volume:

|  | VSLF=Tannk​M28,kρFW | (74) |
| --- | --- | --- |

Land required:

|  | QSLFS DSLF Aacre=VSLF | (75) |
| --- | --- | --- |

Power required:

|  | PWSLF=WSLFspQSLFS | (76) |
| --- | --- | --- |

Composting (CMP)

Unit Specific Parameters

ηOTHCMP: share of OTH counted as degradable

fdegCMP: fraction of VS degraded

αO2CMP: oxygen demand per VS degraded(kg O₂/kg VS)

EARCMP: excess air ratio

yO2, yN2: O₂ and N₂ mass fractions of air

YgCMP: CO₂, H₂O and NH₃ yields per kg of VS degraded plus O₂ consumed  (kg/kg)

EFCH4CMP, EFN2OCMP: CH₄ and N₂O emission factors (kg/kg VS)

θCMP: residence time (h)

ρFW: food waste solids density (kg/m³)

ϵCMP: working volume fraction

WCMPsp: specific power (kW/m³)

Variables

VSinCMP, VSdegCMP: volatile solids entering and degraded (kg/h)

O2CMP: oxygen consumed (kg/h)

ACMP: aeration air (kg/h)

VwCMP, VCMP: working and vessel volumes (m³)

MCMP,COMPOSTP: saleable compost (dry basis) (kg/h)

PWCMP: operating power (kW)

Equations

Volatile solids entering:

|  | VSinCMP=M31,CBH+M31,PRT+M31,FAT+ηOTHCMPM31,OTH | (77) |
| --- | --- | --- |

Volatile solids degraded:

|  | VSdegCMP=fdegCMPVSinCMP | (78) |
| --- | --- | --- |

Oxygen demand:

|  | O2CMP=αO2CMPVSdegCMP | (79) |
| --- | --- | --- |

Aeration air:

|  | ACMP≥EARCMPO2CMPyO2 M33,O2=yO2ACMP M33,N2=yN2ACMP | (80) |
| --- | --- | --- |

Process gas stream :

|  | M32,g=YgCMPVSdegCMP+O2CMP ;g∈{CO2,WATER,NH3} | (81) |
| --- | --- | --- |

Fugitive CH₄ and N₂O:

|  | M32,CH4=EFCH4CMPVSinCMP M32,N2O=EFN2OCMPVSinCMP | (82) |
| --- | --- | --- |

Residual air:

|  | M32,O2=M33,O2-O2CMP, M32,N2=M33,N2 | (83) |
| --- | --- | --- |

Compost organics Stream :

|  | M34,k=1-fdegCMPM31,k ;k∈{CBH,PRT,FAT} | (84) |
| --- | --- | --- |

Compost OTH:

|  | M34,OTH=1-fdegCMPηOTHCMPM31,OTH | (85) |
| --- | --- | --- |

Compost inerts and water:

|  | M34,k=M31,k ;k∈{WATER,ASH,FC} | (86) |
| --- | --- | --- |

Saleable compost (dry basis):

|  | MCMP,COMPOSTP=k∈K​M34,k | (87) |
| --- | --- | --- |

Working volume:

|  | VwCMP=θCMPk​M31,kρFW | (88) |
| --- | --- | --- |

Vessel volume:

|  | ϵCMPVCMP=VwCMP | (89) |
| --- | --- | --- |

Capacity:

|  | QCMPS=VCMP | (89) |
| --- | --- | --- |

Power required:

|  | PWCMP=WCMPspQCMPS | (90) |
| --- | --- | --- |

Wastewater Treatment (WWT)

Unit Specific Parameters

ρww: wastewater density (kg/m³)

fBOD: BOD per kg VS  (kg/kg)

YX: biomass yield (kg/kg)

b: endogenous decay rate  (1/d)

SRT: solids retention time (d)

rkWWT: fraction of component k reporting to sludge

wN: nitrogen mass fraction of the feed

YCO2WWT, YH2OWWT: CO₂ and H₂O yields per VS degraded (kg/kg)

EFCH4WWT, EFN2OWWT: CH₄ and N₂O emission factors (kg/kg VS)

F/M: food to microorganism loading (1/d)

XMLSS: mixed liquor suspended solids (kg/m³)

WWWTsp: specific power (kW/m³)

Variables

QWWT: wastewater flow  (m³/h)

VSinWWT, VSdegWWT: volatile solids entering and degraded (kg/h)

PX: net biosolids yield (kg/h)

NEFF: nitrogen leaving in the effluent  (kg N/h)

mleftWWT: treated effluent (kg/h)

QWWTS: aeration tank volume (m³)

PWWWT: operating power (kW)

Equations

Wastewater flow:

|  | QWWT=k​M35,kρww | (91) |
| --- | --- | --- |

Volatile solids entering:

|  | VSinWWT=k∈KVS​M35,k | (92) |
| --- | --- | --- |

Volatile solids degraded:

|  | VSdegWWT=fBODVSinWWT-YXfBODVSinWWT1+bSRT | (93) |
| --- | --- | --- |

Sludge split stream:

|  | M37,k=rkWWTM35,k ;∀k∈K | (94) |
| --- | --- | --- |

Net biosolids yield:

|  | PX=YXfBODVSinWWT1+bSRT+M37,ASH+M37,FC | (95) |
| --- | --- | --- |

Nitrogen to effluent:

|  | NEFF=wNVSinWWT1-rPRTWWT | (96) |
| --- | --- | --- |

Off-gas CO₂ and water (stream 36):

|  | M36,CO2=YCO2WWTVSdegWWT M36,WATER=YH2OWWTVSdegWWT | (97) |
| --- | --- | --- |

Off-gas CH₄ and N₂O:

|  | M36,CH4=EFCH4WWTVSinWWT M36,N2O=EFN2OWWTVSinWWT | (98) |
| --- | --- | --- |

Treated effluent:

|  | mleftWWT=k​M35,k-k​M37,k-k​M36,k | (99) |
| --- | --- | --- |

Aeration tank volume (F/M basis):

|  | QWWTS=24 fBODVSinWWTF/M XMLSS | (100) |
| --- | --- | --- |

Power required:

|  | PWWWT=WWWTspQWWTS | (101) |
| --- | --- | --- |

Saleable biosolids:

|  | MWWT,BIOSOLIDSP=PX | (102) |
| --- | --- | --- |

Incineration (INC)

Unit Specific Parameters

we: ultimate analysis, e ∈ {C, H, O, N, S}

MWe: atomic weights (kg/kmol)

HHV: feed higher heating value (MJ/kg dry)

hfg: latent heat of water  (MJ/kg)

LHVself: self-sustaining combustion threshold  (MJ/kg wet)

LHVNG, MWNG: natural gas heating value and molecular weight (MJ/kg, kg/kmol)

EFNG: fossil CO₂ from natural gas combustion (kg CO₂/kg)

λINC: excess air ratio

EFCH4INC, EFN2OINC, EFNOxINC: fugitive emission factors (kg/kg wet feed)

WINCsp: specific power (kW per kg/h)

Variables

mdryINC, mashINC, mdafINC: dry, ash and dry ash-free feed  (kg/h)

ne: elemental molar flows(kmol/h)

LHVINC: wet feed lower heating value (MJ/kg)

mNG, nNG: auxiliary natural gas (kg/h, kmol/h)

CO2foss: fossil CO₂ (kg/h)

nO2sto, nO2in, nN2air: stoichiometric O₂, supplied O₂ and air N₂ (kmol/h)

NOx: NOₓ emitted (kg/h)

mflue: flue gas mass (kg/h)

PWINC: operating power  (kW)

Equations

Dry, ash and dry ash-free feed:

|  | mdryINC=k​M38,k-M38,WATER mdafINC=mdryINC-mashINC | (103) |
| --- | --- | --- |

Elemental molar flows:

|  | ne=mdafINC weMWe ;e∈{C,H,O,N,S} | (104) |
| --- | --- | --- |

Wet feed lower heating value:

|  | LHVINCk​M38,k=HHV mdryINC-hfgM38,WATER | (105) |
| --- | --- | --- |

Fuel to reach the self-sustaining threshold:

|  | mNG LHVNG≥LHVself-LHVINCk​M38,k | (106) |
| --- | --- | --- |

Natural gas moles and fossil CO₂:

|  | nNG=mNGMWNG, CO2foss=EFNGmNG | (107) |
| --- | --- | --- |

Stoichiometric oxygen:

|  | nO2sto=nC+0.25nH+nS-0.5nO+2nNG | (108) |
| --- | --- | --- |

Combustion air:

|  | nO2in=λINCnO2sto nN2air=0.790.21nO2in | (109) |
| --- | --- | --- |

Combustion products:

|  | nCO2=nC+nNG, nH2O=0.5nH+2nNG nSO2=nS | (110) |
| --- | --- | --- |

Excess oxygen and nitrogen:

|  | nO2out=nO2in-nO2sto nN2out=nN2air+0.5nN | (111) |
| --- | --- | --- |

Fugitive emissions (stream 39):

|  | M39,CH4=EFCH4INCk​M38,k M39,N2O=EFN2OINCk​M38,k | (112) |
| --- | --- | --- |

NOₓ emitted:

|  | NOx=EFNOxINCk​M38,k | (113) |
| --- | --- | --- |

Flue gas CO₂, water and SO₂ stream:

|  | M40,CO2=44.01 nCO2 M40,WATER=M38,WATER+18.015 nH2O M40,SO2=64.06 nSO2 | (114) |
| --- | --- | --- |

Flue gas O₂ and N₂:

|  | M40,O2=32.00 nO2out M40,N2=28.0134 nN2out | (115) |
| --- | --- | --- |

Flue gas mass:

|  | mflue=k​M40,k | (116) |
| --- | --- | --- |

Capacity:

|  | QINCS=k​M38,k | (117) |
| --- | --- | --- |

Power  required:

|  | PWINC=WINCspQINCS | (117) |
| --- | --- | --- |

Centrifuge (CEN)

Unit Specific Parameters

ηCEN: separation efficiency

ρk: density of HTL product

WCENsp: specific power  (kW per m³/h)

Cp,w: heat capacity of cooling water (kJ/(kg K))

Tincw, Toutcw: cooling water supply and return temperatures  (°C)

Variables

CFCEN: volumetric concentration factor

UCEN: sigma factor

QCENS: equivalent settling area (sigma) (m²)

PWCEN: operating power (kW)

MCENCW : cooling water (kg/h)

Equations

Component balance:

|  | M41,k=M42,k+M43,k ;∀k∈K | (118) |
| --- | --- | --- |

Liquid product stream:

|  | M43,k=ηCENM41,k ;∀k∈K | (119) |
| --- | --- | --- |

Char cake stream:

|  | M42,CHAR=ηCENM41,CHAR | (120) |
| --- | --- | --- |

Concentration factor:

|  | CFCENM42,CHARρCHAR=k∈KHTL​M41,kρk | (121) |
| --- | --- | --- |

Sigma factor:

|  | QCENSUCEN=k∈KHTL​M41,kρk | (122) |
| --- | --- | --- |

Power required:

|  | PWCEN=WCENspk∈KHTL​M41,kρk | (123) |
| --- | --- | --- |

Power dissipated as heat is about 40%, so cooling duty is required:

|  | MCENCWCp,wToutcw-Tincw=0.4×3600 PWCEN | (124) |
| --- | --- | --- |

Filtration (FLT)

Unit Specific Parameters

RkFLT: fraction of HTL product k retained by the filter

ρk: density of HTL product k (kg/m³)

ζFLT: permeate flux (m³/(h m²))

WFLTsp; specific power (kW/m²)

Variables

CFFLT: volumetric concentration factor

QFLTS: filter area (m²)

PWFLT: operating power (kW)

Equations

Retentate (stream 45):

|  | M45,k=RkFLTM44,k ;∀k∈KHTL | (125) |
| --- | --- | --- |

Permeate (stream 46):

|  | M46,k=1-RkFLTM44,k ;∀k∈KHTL | (126) |
| --- | --- | --- |

Concentration factor:

|  | CFFLTk∈KHTL​M45,kρk=k∈KHTL​M44,kρk | (127) |
| --- | --- | --- |

Flux balance:

|  | ζFLTQFLTSCFFLT=k∈KHTL​M44,kρkCFFLT-1 | (128) |
| --- | --- | --- |

Power required:

|  | PWFLT=WFLTspQFLTS | (129) |
| --- | --- | --- |

Absorption (ABS)

Unit Specific Parameters

sCH4ABS: CH₄ slip to offgas

ηCO2ABS: CO₂ removal

Vm: molar volume at STP (m³/kmol)

MWCH4, MWCO2: molecular weights  (kg/kmol)

eABS: electricity per Nm³ of biogas (kWh/Nm³)

xpur: required CH₄ purity (mol/mol)

rwABS: wash water per kg CO₂ removed (kg/kg CO₂)

sstm: amine regeneration steam per Nm³ of biogas  (kg/Nm³)

Variables

QABSS: biogas flow treated (Nm³/h)

PWABS: operating power (kW)

MABSSTM: regeneration steam (kg/h)

Equations

Biomethane product stream:

|  | M52,CH4=1-sCH4ABSM49,CH4, M52,CO2=1-ηCO2ABSM49,CO2 | (130) |
| --- | --- | --- |

Off-gas stream:

|  | M51,CH4=sCH4ABSM49,CH4, M51,CO2=ηCO2ABSM49,CO2 | (131) |
| --- | --- | --- |

Wash water stream:

|  | M50,WATER=rwABSηCO2ABSM49,CO2 | (132) |
| --- | --- | --- |

Biogas flow at STP:

|  | QABSS=VmM49,CH4MWCH4+M49,CO2MWCO2 | (133) |
| --- | --- | --- |

Power required:

|  | PWABS=eABSQABSS | (134) |
| --- | --- | --- |

Regeneration steam:

|  | MABSSTM=sstmQABSS | (135) |
| --- | --- | --- |

Product purity:

|  | 1-xpurM52,CH4MWCH4≥xpurM52,CO2MWCO2 | (136) |
| --- | --- | --- |

Saleable biomethane:

|  | MABS,CH4P=M52,CH4 | (137) |
| --- | --- | --- |

Pressure Swing Adsorption (PSA)

Unit Specific Parameters

sCH4PSA: CH₄ slip to purge

ηCO2PSA: CO₂ removal

Vm: molar volume at STP (m³/kmol)

MWCH4, MWCO2: molecular weights (kg/kmol)

ePSA: electricity per Nm³ of biogas  (kWh/Nm³)

xpur: required CH₄ purity (mol/mol)

Variables

QPSAS: biogas flow treated  (Nm³/h)

PWPSA: operating power  (kW)

Equations

Biomethane product stream:

|  | M55,CH4=1-sCH4PSAM53,CH4 M55,CO2=1-ηCO2PSAM53,CO2 | (138) |
| --- | --- | --- |

Off-gas stream:

|  | M54,CH4=sCH4PSAM53,CH4, M54,CO2=ηCO2PSAM53,CO2 | (139) |
| --- | --- | --- |

Biogas flow at STP:

|  | QPSAS=VmM53,CH4MWCH4+M53,CO2MWCO2 | (140) |
| --- | --- | --- |

Power required:

|  | PWPSA=ePSAQPSAS | (141) |
| --- | --- | --- |

Product purity:

|  | 1-xpurM55,CH4MWCH4≥xpurM55,CO2MWCO2 | (142) |
| --- | --- | --- |

Saleable biomethane:

|  | MPSA,CH4P=M55,CH4 | (143) |
| --- | --- | --- |

Steam Turbine (STB)

Unit Specific Parameters

Cp,g: flue gas heat capacity  (kJ/(kg K))

Ting, Toutg: flue gas inlet and outlet temperatures (°C)

H1, H2: feedwater and steam enthalpies  (kJ/kg)

h1, h2: turbine inlet and outlet enthalpies (kJ/kg)

ηt, ηg: turbine and generator efficiencies

WSTBsp: specific parasitic power (kW/kW)

Variables

M59,WATER: boiler feedwater raised to steam (kg/h)

Pturb: turbine shaft work  (kJ/h)

QSTBS: net electricity generated  (kW)

PWSTB: power  (kW)

Equations

Heat recovery from the flue gas:

|  | mflueCp,gTing-Toutg=M59,WATERH2-H1 | (144) |
| --- | --- | --- |

Steam raised (stream 60):

|  | M60,WATER=M59,WATER | (145) |
| --- | --- | --- |

Turbine shaft work:

|  | Pturb=M60,WATERh1-h2 | (146) |
| --- | --- | --- |

Electricity generated:

|  | QSTBS=ηtηgPturb3600 | (147) |
| --- | --- | --- |

Power required:

|  | PWSTB=WSTBspQSTBS | (148) |
| --- | --- | --- |

Saleable electricity (stream 57):

|  | M57,ELECTRICITY=MSTB,ELECTRICITYP=QSTBS | (149) |
| --- | --- | --- |

S3. Costing

S3.1. Cost Variables

CCAP - Annualized capital cost (CCAC), M$/yr

CWC - Annualized working capital (CCWC), M$/yr

CINS - Insurance and property tax (CCINS), M$/yr

CLBR - Labor cost (CCLB), M$/yr

CUTL - Utility cost (CCUC), M$/yr

CFAC - Facility-dependent cost: maintenance, overhead and local taxes (CCOC), M$/yr

CRM - Raw material cost (CCRM), M$/yr

CDSP - Disposal cost (CDISP), M$/yr

CTIP - Tipping fee received (CTIP), M$/yr

CTC - Total annual cost (CCTC), M$/yr

Rev - Annual revenue, product sales plus tipping fee (REV), M$/yr

S3.2. Cost Equations

Purchase cost for technology i

|  | CiPC=Ci0QiCQi0nc ;i∈I | (150) |
| --- | --- | --- |

where Ci0 and Qi0 are the reference cost and capacity and nc the six-tenths scaling exponent

Annualized capital,

|  | CCAP=CRF FBMi​CiPC106 | (151) |
| --- | --- | --- |

working capital,

|  | CWC=rWCCRF FBMi​CiPC106 | (152) |
| --- | --- | --- |

insurance

|  | CINS=rINSFBMi​CiPC106 | (153) |
| --- | --- | --- |

facility-dependent cost

|  | CFAC=rFACFBMi​CiPC106 | (154) |
| --- | --- | --- |

Labor cost

|  | NiLBQi0=NirQiC ;i∈I CLBR=cLB Tanni​NiLB106 | (155) |
| --- | --- | --- |

Utility cost

|  | CUTL=Tann106i​celPWi+cstmMiSTM+ccwMiCW | (156) |
| --- | --- | --- |

Raw material cost

|  | CRM=Tann106(cwM7,W+M21,W+M24,W+M59,W +cenzM15,ENZYME+camλamM50,W+cNGmNG) | (157) |
| --- | --- | --- |

Disposal cost

|  | CDSP=Tann106(crejk​M5,k+ccharM42,CHAR+M45,CHAR | (158) |
| --- | --- | --- |
|  | +caqM42,AQ+M45,AQ+M50,W +clandk​M28,k+mleftWWT+mashINC) |  |

Tipping fee

|  | CTIP=ctip Tann106k​M1,k | (159) |
| --- | --- | --- |

Revenue

|  | Rev=Tann106(pCH4M56,CH4+pBIOM47,BIOCRUDE+pCMPMCMP,COMPOSTP +pBSPX+pELQSTBS)+CTIP | (160) |
| --- | --- | --- |

Total annual cost

|  | CTC=CCAP+CWC+CINS+CUTL+CLBR+CFAC+CRM+CDSP | (161) |
| --- | --- | --- |

Net annual cost

|  | NAC=CTC-Rev | (162) |
| --- | --- | --- |

where CRF is the capital recovery factor, FBM the bare-module multiplier, r the working capital, insurance and facility cost ratios, c the unit costs and p the product prices (Tables S2.6 and S2.7). Digestate is land applied and carries no disposal cost.

S4. Environmental impact equations

Indirect emissions from electricity and steam

|  | GHGIND=Tann1000EFeli​PWi+EFstmi​MiSTM | (163) |
| --- | --- | --- |

Direct emissions

|  | GHGDIR=Tann1000ECH4+EN2O+CO2foss+EHTL ECH4=GWPCH4M25,CH4+M29,CH4+M32,CH4+M36,CH4+M39,CH4 EN2O=GWPN2OM32,N2O+M36,N2O+M39,N2O EHTL=EFHTLgas+GWPCH4ϕventwCH4gasM20,GAS | (164) |
| --- | --- | --- |

Displacement credits

|  | GHGCR=Tann1000(EFCH4dM56,CH4+EFBIOdM47,BIOCRUDE +EFCMPdMCMP,COMPOSTP+EFBSdPX+EFELdQSTBS) | (165) |
| --- | --- | --- |

HTL residue burdens and digestate credit

|  | GHGAQ=Tann1000EFaqM42,AQ+M45,AQ+EFcharM42,CHAR+M45,CHAR GHGDIG=Tann1000 EFdigk≠WATER​M26,k | (166) |
| --- | --- | --- |

Net greenhouse gas emissions

|  | GHGTOT=GHGIND+GHGDIR+GHGAQ-GHGCR-GHGDIG | (167) |
| --- | --- | --- |

Marine eutrophication potential

|  | EPMAR=Tann1000NAQ+NEFF+u∈{CMP,AD,SLF}​λuleachwNk∈KVS​Mju,k+CFNOxNOx | (168) |
| --- | --- | --- |

where ju is the stream carrying the residue that leaches: 34 (compost), 26 (digestate) and 28 (landfilled feed). The emission factors are listed in Table S4.1-B. ϕvent is the fraction of HTL gas assumed vented and wCH4gas its CH₄ mass fraction. The EPMAR characterization factor is 1 kg N-eq per kg N for nitrogen to water.

S5. Model parameters

Table S1-3 lists the measured analyses and the wet-basis component fractions used in the feed equation. Table S2.4 lists the feed dependent yields computed from these analyses.

Table S1: Proximate analysis of food waste feed.

| Property | Value | Basis |
| --- | --- | --- |
| Moisture | 75.95 | % wet |
| Volatile matter | 80.23 | % dry |
| Ash | 5.17 | % dry |
| Fixed carbon | 14.60 | % dry |

Table S2: Biochemical composition of food waste feed.

| Property | Value | Basis |
| --- | --- | --- |
| Carbohydrate | 6.95 | % dry |
| Protein | 44.82 | % dry |
| Lipid | 19.28 | % dry |

Table S3: Ultimate analysis of food waste feed.

| Property | Value | Basis |
| --- | --- | --- |
| Carbon | 43.27 | % wet |
| Hydrogen | 6.05 | % dry |
| Nitrogen | 7.17 | % dry |
| Oxygen | 42.89 | % dry |
| Sulfur | 0.62 | % dry |

Table S4: Feed-dependent yields computed in the feed characterization

| Quantity | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Buswell AD yield, CH₄ | YCH4AD | 0.2703 | kg/kg VS destroyed |
| Buswell AD yield, CO₂ | YCO2AD | 0.8441 | kg/kg VS destroyed |
| Buswell AD yield, NH₃ | YNH3AD | 0.0872 | kg/kg VS destroyed |
| Buswell AD yield, H₂S | YH2SAD | 0.0066 | kg/kg VS destroyed |
| Water consumed by hydrolysis | YH2OAD | 0.2081 | kg/kg VS destroyed |
| HTL biocrude, | YBIOHTL | 0.3523 | kg/kg organics |
| HTL char, | YCHARHTL | 0.1023 | kg/kg organics |
| HTL gas, | YGASHTL | 0.1479 | kg/kg organics |
| HTL aqueous phase, by difference | YAQHTL | 0.3974 | kg/kg organics |

Buswell yields use the feed formula C1HhOoNnSs from the ultimate analysis. The HTL yields depend on z, the dry-basis protein plus lipid fraction (z=0.641 for the feed basis). The linear yield coefficients were calibrated so that a 50/50 feed reproduces biocrude, char, aqueous and gas yields of 0.35, 0.10, 0.40 and 0.15.

Table S5: Capital cost, power and labor correlation data by technology

| Technology | Ci0 ($) | Qi0 | Capacity unit | Wisp (kW/unit) | Nir (operators) |
| --- | --- | --- | --- | --- | --- |
| SHR | 111,000 | 10,000 | kg/h | 0.02 | 0.1 |
| MCR | 111,000 | 60,000 | kg/h | 0.10 | 0.1 |
| AER | 882,000 | 15,000 | m³ | 0.04 | 0.5 |
| ENZ | 882,000 | 15,000 | m³ | 0.04 | 0.5 |
| HTL | 645,000 | 40 | m³ | 2.0 | 2.0 |
| AD | 594,000 | 1,000 | m³ | 0.005 | 0.02 |
| SLF | 450,000 | 0.15 | acre | 0.5 | 1.0 |
| CMP | 786,000 | 350 | m³ | 0.02 | 0.5 |
| WWT | 8,000,000 | 15,000 | m³ | 0.04 | 0.5 |
| INC | 4,700,000 | 8,000 | kg/h | 0.05 | 1.0 |
| CEN | 66,000 | 0.01 | m² (sigma) | 0.1 | 1.0 |
| FLT | 39,000 | 80 | m² | 0.1 | 0.5 |
| ABS | 30,000 | 32 | Nm³/h | 0.1 | 0.01 |
| PSA | 80,000 | 50 | Nm³/h | 0.4 | 0.01 |
| STB | 45,000 | 30,000 | kW | 0.02 | 0.05 |